IoT PROJECT SUMMARY

Server Room Safety Monitoring System

IoT Course Project

2025 / 2026

Students: Svetlana Gavris

Shirin Fakiri

1. Project Overview

Server Room Safety is a complete IoT monitoring system designed to continuously track environmental conditions inside a server room and respond automatically to dangerous situations. The system uses MQTT as its messaging backbone, Python-based sensor emulators as data producers, a SQLite database for persistence, and a Tkinter GUI for real-time visualization.

1.1 Motivation

Server rooms are mission-critical environments. Overheating, smoke, high CO2 levels, or unauthorized access can cause expensive hardware damage or data loss. A real-time IoT monitoring system provides immediate detection and automated responses, significantly reducing response times.

1.2 Key Features

- Three types of sensor/actuator emulators (DHT22, MQ-2/MQ-135, Door+Fan relay)

- MQTT-based pub/sub communication via Mosquitto broker

- Automated threshold evaluation with WARNING and ALARM levels

- Auto-remediation: fan and cooling relay activate on high temperature/smoke

- SQLite database for all sensor readings, device states, and alerts

- Real-time Tkinter dashboard with live charts and alerts log

- Manual relay control from the GUI

- Multi-scenario simulation (normal, heating, cooling, smoke, critical)

2. System Architecture

The system is organized into four independent layers that communicate via MQTT. This decoupled architecture allows any component to be replaced or scaled independently.

2.1 Architecture Diagram

┌─────────────────────────────────────────────────────────────────┐

│                      EMULATORS (Producers)                      │

│  dht_emulator.py  │  smoke_emulator.py  │  button_emulator.py  │

│  (Temp/Humidity)  │  (Smoke/CO2 + Relay) │  (Door/Fan Control)  │

└────────────────────────────┬────────────────────────────────────┘

│ MQTT (localhost:1883)

┌────────────────────────────▼────────────────────────────────────┐

│                      DATA MANAGER APP                           │

│  data_manager.py — Collect, store to SQLite, send Alerts        │

└────────────────────────────┬────────────────────────────────────┘

│ MQTT + SQLite

┌────────────────────────────▼────────────────────────────────────┐

│                        MAIN GUI APP                             │

│  gui_app.py — Live dashboard, charts, alerts, controls          │

└─────────────────────────────────────────────────────────────────┘

2.2 MQTT Topic Map

| Topic | Direction | Description |
| --- | --- | --- |
| serverroom/temperature | EMU → DM, GUI | Temperature in °C |
| serverroom/humidity | EMU → DM, GUI | Humidity in % |
| serverroom/smoke | EMU → DM, GUI | Smoke level in ppm |
| serverroom/co2 | EMU → DM, GUI | CO2 level in ppm |
| serverroom/door | EMU → DM, GUI | Door OPEN / CLOSED |
| serverroom/fan | EMU → DM, GUI | Fan relay state + speed |
| serverroom/cooling | EMU → DM, GUI | Cooling relay state |
| serverroom/alert/warning | DM → GUI | Warning alert message |
| serverroom/alert/alarm | DM → GUI | Critical alarm message |
| serverroom/control/fan | GUI / DM → EMU | Fan relay command |
| serverroom/control/cooling | GUI / DM → EMU | Cooling relay command |

3. Project Components

3.1  Emulators — Data / Message Producers

3.1.1  DHT22 Emulator (dht_emulator.py)

Simulates a DHT22 temperature and humidity sensor. Publishes readings every 2 seconds using a sinusoidal drift + Gaussian noise model. Supports four simulation scenarios: normal, heating, cooling, and critical.

- Topic published: serverroom/temperature, serverroom/humidity

- Payload: JSON with sensor name, value, unit, and tick counter

- Scenarios: normal (22°C base), heating (rises to ~40°C), cooling (drops to ~14°C), critical (37°C+)

3.1.2  Smoke & CO2 Emulator + Cooling Relay (smoke_emulator.py)

Simulates MQ-2 (smoke) and MQ-135 (CO2) sensors with configurable scenarios. Also acts as a relay actuator, subscribing to cooling control commands and reporting the relay state back to the broker.

- Topics published: serverroom/smoke, serverroom/co2, serverroom/cooling

- Topic subscribed: serverroom/control/cooling

- Scenarios: normal (50 ppm smoke / 600 ppm CO2), smoke (rising to alarm), critical

3.1.3  Door Button + Fan Relay Emulator (button_emulator.py)

Simulates a door contact sensor (button) and a manual fan knob, plus a fan relay actuator. The door randomly opens for a few ticks then closes. The knob value drifts using Gaussian noise. The fan relay responds to MQTT control commands.

- Topics published: serverroom/door, serverroom/fan, serverroom/fan_speed

- Topic subscribed: serverroom/control/fan

- Door: 5% probability of opening each tick, stays open 2–6 ticks

3.2  Data Manager (data_manager.py)

The central intelligence of the system. Subscribes to all sensor topics, persists data to SQLite, evaluates thresholds, and publishes WARNING or ALARM messages. Also implements auto-remediation by sending relay commands when thresholds are breached.

3.2.1  Alert Thresholds

| Sensor | Unit | Warning | Alarm |
| --- | --- | --- | --- |
| Temperature | °C | > 28 °C | > 35 °C → Fan 100% + Cooling ON |
| Humidity | % | > 70 % | > 85 % → Alert only |
| Smoke | ppm | > 200 ppm | > 500 ppm → Fan 100% |
| CO2 | ppm | > 1000 ppm | > 2000 ppm → Alert only |

3.2.2  Alert Deduplication

The Data Manager tracks the last alert level per sensor and only publishes a new alert when the level changes (e.g., from OK to WARNING, or WARNING to ALARM). This prevents flooding the broker and the GUI with repeated messages.

3.3  Main GUI Dashboard (gui_app.py)

A Tkinter-based real-time dashboard with an embedded Matplotlib chart area. The GUI subscribes to all MQTT topics in a background thread and refreshes the UI every 1 second using Tkinter's after() scheduler.

3.3.1  GUI Components

- Title bar: connection status indicator and live clock

- Gauge row: current value for each sensor with color-coded status (OK / WARNING / ALARM)

- Charts: 2×2 grid of scrolling line charts, one per sensor, with threshold lines

- Device status panel: real-time Fan, Cooling, and Door state display

- Manual controls: buttons for Fan ON/OFF and Cooling ON/OFF

- Alerts log: scrollable text widget showing last 30 alerts, color-coded by level

3.3.2  Thread Safety

All MQTT callbacks write to a shared AppState object protected by a threading.Lock(). The Tkinter refresh function reads from the same object in the main thread, ensuring no race conditions between the MQTT thread and the UI thread.

4. Local Database (SQLite)

SQLite is used as the local database, automatically initialized by the Data Manager on first run. The database file is stored at db/serverroom.db.

4.1 Schema

| Table | Columns | Description |
| --- | --- | --- |
| sensor_readings | id, timestamp, sensor_type, value, unit | All numeric sensor readings |
| device_states | id, timestamp, device, state, extra | Fan / cooling / door state changes |
| alerts | id, timestamp, level, sensor_type, value, message | WARNING and ALARM events |
| event_log | id, timestamp, event_type, description | General event log (optional) |

4.2 DB Utilities

The project includes two database utilities:

- db/init_db.py — Standalone script to initialize the database and print the schema

- db/db_query.py — CLI inspector: summary, latest readings, alerts, device changes, CSV export

5. Running the Project

5.1 Prerequisites

- Python 3.9+

- Mosquitto MQTT broker installed and running

- Python packages: pip install paho-mqtt matplotlib

5.2 Quick Start (Single Command)

Use the all-in-one launcher:

python run_all.py

python run_all.py --scenario heating    # high temperature scenario

python run_all.py --scenario smoke      # smoke alarm scenario

python run_all.py --no-gui              # headless / server mode

5.3 Manual Start (4 Terminals)

Terminal 1 — MQTT Broker:

mosquitto

Terminal 2 — Data Manager:

python data_manager/data_manager.py

Terminal 3 — GUI:

python gui/gui_app.py

Terminal 4 — Emulators:

python emulators/dht_emulator.py --scenario heating &

python emulators/smoke_emulator.py &

python emulators/button_emulator.py

6. Project File Structure

ServerRoomSafety/

├── emulators/

│   ├── dht_emulator.py      # DHT22 temperature & humidity emulator

│   ├── smoke_emulator.py    # Smoke / CO2 sensor + cooling relay

│   └── button_emulator.py   # Door contact + knob + fan relay

├── data_manager/

│   └── data_manager.py      # MQTT subscriber, DB writer, alert engine

├── gui/

│   └── gui_app.py           # Tkinter real-time dashboard

├── db/

│   ├── init_db.py           # DB schema initializer

│   ├── db_query.py          # CLI DB inspector / CSV exporter

│   └── serverroom.db        # SQLite database (auto-created)

├── run_all.py               # All-in-one launcher

├── requirements.txt

└── README.md

7. Grading Criteria Mapping

| Criterion | Points | Implementation |
| --- | --- | --- |
| ≥3 emulator types (DHT, Relay, Button/Knob) | 6 pts | dht_emulator, smoke_emulator, button_emulator |
| Data Manager (DB + Warning/Alarm messages) | 8 pts | data_manager.py + SQLite |
| Main GUI (live data + Info/Warning/Alarm) | 10 pts | gui_app.py with charts & alerts log |
| Local / Cloud DB | 3 pts | SQLite in db/serverroom.db |
| Project Summary (docx + pdf) | 3 pts | This document |

8. Conclusion

The Server Room Safety project demonstrates a complete, production-quality IoT pipeline from sensor data generation to persistent storage and real-time visualization. All required components are implemented: three emulator types covering both data producers and actuators, a data manager with threshold-based alerting and auto-remediation, a full GUI dashboard, and a local SQLite database.

The MQTT-based architecture ensures that all components are loosely coupled and can be independently scaled, replaced, or extended — for example, by substituting real hardware sensors for the emulators or by adding a cloud database backend.